| Normal Mode | LED Display | Failure Mode | LED Display "RED Flashing" | Possible Repair Action |
| --- | --- | --- | --- | --- |
| Powered Up and No Controller Connected (ALL LEDS “OFF”) |  | AV Cable not connected to console (ALL LEDS “ON”) |  | 1. Connect AV cable to console |
| Powered Up and 1 Controller Connected |  | Thermal Overload |  | 1. Check for proper fan operation and restricted airflow. Disconnect power supply and wait a couple of minutes. Reconnect power supply and restart. If console fails again, note the error code then return for repair |
|  |  | Core Digital or System Component Failure |  | 1. Disconnect power supply and wait a couple of minutes. Reconnect power supply and restart. If console fails again, note the error code then return for repair |
|  |  | Universal Error Message on screen |  | 1. See UEM Error codes sheet. |

| UEM ERROR CODES |  |  |  |  |
| --- | --- | --- | --- | --- |
| VALUE | 0 | 1 | 2 | 3 |
| DISPLAY |  |  |  |  |
|  | (ALL LEDS “ON”) | (1 LED “ON”) | (2 LEDS “ON”) | (3 LEDS “ON”) |

| SMC (System Management Controller) |  |  |  |  |
| --- | --- | --- | --- | --- |
| ERROR CODE |  | ERROR NAME | DESCRIPTION | POSSIBLE ACTIONS |
| LED FRONT PANEL | TV SCREEN MONITOR |  |  |  |
| 0001 |  | ERROR_V_12P0 | ANA_V12P0_PWRGD was negated unexpectedly | Replace the power supply. If console still displays error, then note error code and return console for repair. |
| 0002 |  | ERROR_V_CPUCORE | VREG_CPU_PWRGD was negated unexpectedly | Replace the power supply. If console still displays error, then note error code and return console for repair. |
| 0003 |  | ERROR_V_GPUCORE | VREG_GPU_PWRGD was negated unexpectedly | Replace the power supply. If console still displays error, then note error code and return console for repair. |
| 0010 |  | ERROR_NO_ANA | Ana is not responding to reads/writes | Note error code and return console for repair |
| 0011 |  | ERROR_THERMAL_CPU | A thermal overload occurred in the CPU | Check console for blocked or restricted air flow. |
| 0012 |  | ERROR_THERMAL_GPU | A thermal overload occurred in the GPU | Check console for blocked or restricted air flow. |
| 0013 |  | ERROR_THERMAL_EDRAM | A thermal overload occurred in the EDRAM | Check console for blocked or restricted air flow. |
| 0020 |  | ERROR_GPU_RST_DONE | GPU_RST_DONE was not asserted in time during seqUnReset | Note error code and return console for repair |
| 0021 |  | ERROR_NO_PCIE | The PCI-E link failed to enter L0 in time during seqUnReset | Replace the power supply. If console still displays error, then note error code and return console for repair. |
| 0022 |  | ERROR_NO_HANDSHAKE | The CPU failed to send GetPowerUpCause to the SMC. Also used for Titan retries. | Note error code and return console for repair. |
| 0023 |  | ERROR_NO_CLOCKCHIP | Backup clock chip (ICS/Cypress) is not responding to reads or writes (only in Retail mode) | Note error code and return console for repair |
| 0030 |  | ERROR_NO_TEMPERATURES | Ana responds to temperature reads, but fails to provide temperature samples (registers read as zero). | Note error code and return console for repair |
| 0031-0033 |  | RESERVED_0x0D_0x0F | Not used |  |
| XSS (XBox System Software) |  |  |  |  |
| ERROR CODE |  | ERROR NAME | DESCRIPTION | POSSIBLE ACTIONS |
| LED FRONT PANEL | TV SCREEN MONITOR |  |  |  |
| 0101 | E17 | ERROR_NBINIT_MEM_READ_STROBE_DATA_WRITE | Unable to write data for read strobe training | Note error code and return console for repair |
| 0102 | E18 | ERROR_NBINIT_MEM_READ_STROBE_DELAY_TRAINING | Memory read strobe delay training failed | Note error code and return console for repair |
| 0103 | E19 | ERROR_NBINIT_MEM_WRITE_STROBE_DELAY_TRAINING | Memory write strobe delay training failed | Note error code and return console for repair |
| 0110 | E20 | ERROR_MEMORY_ADDRESSING | Memory address line failure | Note error code and return console for repair |
| 0111-0133 |  | RESERVED_0x15_0x1F | Reserved |  |
| 0200-0233 |  | RESERVED_0x20_0x2F | Reserved |  |
| 0300 | E48 | ERROR_MFG_COMM_FAILURE |  | Note error code and return console for repair |
| 0301 | E49 | ERROR_MFG_UNABLE_TO_CONNECT |  | Note error code and return console for repair |
| 0302 | E50 | ERROR_MFG_CONNECTX_FAILURE |  | Note error code and return console for repair |
| 0303 | E51 | ERROR_MFG_GATEWAY_ERROR |  | Note error code and return console for repair |
| 0310 | E52 | ERROR_MFG_BAD_RESPONSE_DATA |  | Note error code and return console for repair |
| ERROR CODE |  | ERROR NAME | DESCRIPTION | POSSIBLE ACTIONS |
| LED FRONT PANEL | TV SCREEN MONITOR |  |  |  |
| 0311 | E53 | ERROR_MFG_CHALLENGE_REFUSED |  | Note error code and return console for repair |
| 0312 | E54 | ERROR_MFG_BAD_VAULT |  | Note error code and return console for repair |
| 0313 |  | ERROR_XSS_MANUF_TEST[8_16] | Manufacturing test error code | Note error code and return console for repair |
| 1000 | E64 | ERROR_XSS_CDROM_TIMED_OUT | Kernel communications to ODD timed out | Note error code and return console for repair |
| 1001 | E65 | ERROR_XSS_CDROM_NOT_DMA_CONFIGURED | Kernel was not able to configure ODD for DMA mode | Note error code and return console for repair |
| 1002 | E66 | ERROR_XSS_CDROM_COULD_NOT_CREATE_DEVICE | Kernel wasn’t able to instantiate ODD | Note error code and return console for repair |
| 1003 | E67 | ERROR_XSS_HDD_TIMED_OUT | Kernel communications to HDD timed out | Note error code and return console for repair |
| 1010 | E68 | ERROR_XSS_HDD_NOT_DMA_CONFIGURED | Kernel was not able to configure HDD for DMA mode | Note error code and return console for repair |
| 1011 | E69 | ERROR_XSS_HDD_COULD_NOT_CREATE_DEVICE | Kernel wasn’t able to instantiate HDD | Note error code and return console for repair |
| 1012 | E70 | ERROR_XSS_HDD_NOT_FOUND | Kernel couldn’t find the HDD | Note error code and return console for repair |
| 1013 | E71 | ERROR_XSS_XAM_NO_DASH | XAM could not find dash to load | Note error code and return console for repair |
| 1020 | E72 | ERROR_SMC_FLASH_CONFIG_CORRUPT | SMC configuration area in flash is corrupt | Note error code and return console for repair |
| 1021 | E73 | ERROR_HSIO_TRAINING_FAILED | GPU high speed link failed | Note error code and return console for repair |
| 1022 | E74 | ERROR_HSIO_CALIBRATION_FAILED | GPU high speed link failed | Note error code and return console for repair |
| 1023 | E75 | ERROR_UNEXPECTED_ETHERNET_PHY_VENDOR | Could not talk to Ethernet PHY over MDIO/MDC | Note error code and return console for repair |
| 1030 | E76 | ERROR_ETHERNET_PHY_RESET_FAIL | Ethernet PHY failed to complete reset | Note error code and return console for repair |
| ERROR CODE |  | ERROR NAME | DESCRIPTION | POSSIBLE ACTIONS |
| LED FRONT PANEL | TV SCREEN MONITOR |  |  |  |
| 1031 | E77 | ERROR_ETHERNET_PHY_READ_WRITE_FAIL | Could not read and/or write to the Ethernet PHY | Note error code and return console for repair |
| 1032 | E78 | ERROR_UNSUPPORTED_GPU | GPU is down rev. and not supported | Note error code and return console for repair |
| 1033 | E79 | ERROR_XSS_FLASH_NO_XAM | XAM could not be found in the flash | Note error code and return console for repair |
| 1100-2000 |  | RESERVED 0x50 0x80 | Reserved |  |
| 2001-2120 |  | ERROR_POST_1BL_W2_E_RESERVED |  | Note error code and return console for repair |
| 2121-2122 |  | RESERVED 0x99 0x9A | Reserved |  |
| 2123 |  | ERROR_POST_2BL_E_VERIFY_SECOTP_1 |  | Note error code and return console for repair |
| 2130 |  | ERROR_POST_2BL_E_VERIFY_SECOTP_2 |  | Note error code and return console for repair |
| 2131 |  | ERROR_POST_2BL_E_VERIFY_SECOTP_3 |  | Note error code and return console for repair |
| 2132 |  | ERROR_POST_2BL_E_VERIFY_SECOTP_4 |  | Note error code and return console for repair |
| 2133 |  | ERROR_POST_2BL_E_VERIFY_SECOTP_5 |  | Note error code and return console for repair |
| 2200 |  | ERROR_POST_2BL_E_VERIFY_SECOTP_6 |  | Note error code and return console for repair |
| 2201 |  | ERROR_POST_2BL_E_VERIFY_SECOTP_7 |  | Note error code and return console for repair |
| 2202 |  | ERROR_POST_2BL_E_VERIFY_SECOTP_8 |  | Note error code and return console for repair |
| 2203 |  | ERROR_POST_2BL_E_VERIFY_SECOTP_9 |  | Note error code and return console for repair |
| 2210 |  | RESERVED_0xA4 | Reserved |  |
| 2211 |  | ERROR_POST_2BL_E_VERIFY_OFFSET_3BL |  | Note error code and return console for repair |
| ERROR CODE |  | ERROR NAME | DESCRIPTION | POSSIBLE ACTIONS |
| LED FRONT PANEL | TV SCREEN MONITOR |  |  |  |
| 2212 |  | ERROR_POST_2BL_E_LOCATE_3BL |  | Note error code and return console for repair |
| 2213 |  | ERROR_POST_2BL_E_VERIFY_HEADER_3BL |  | Note error code and return console for repair |
| 2220 |  | ERROR_POST_2BL_E_SIG_VERIFY_3BL |  | Note error code and return console for repair |
| 2221 |  | ERROR_POST_2BL_E_HWINIT |  | Note error code and return console for repair |
| 2222 |  | ERROR_POST_2BL_E_VERIFY_OFFSET_4BL |  | Note error code and return console for repair |
| 2223 |  | ERROR_POST_2BL_E_VERIFY_HEADER_4BL |  | Note error code and return console for repair |
| 2230 |  | ERROR_POST_2BL_E_SIG_VERIFY_4BL |  | Note error code and return console for repair |
| 2231 |  | ERROR_POST_2BL_E_SHA_VERIFY_4BL |  | Note error code and return console for repair |
| 2232 |  | ERROR_POST_2BL_E_UNEXPECTED_INTERRUPT |  | Note error code and return console for repair |
| 2233 |  | ERROR_POST_2BL_E_UNSUPPORTED_RAM_SIZE |  | Note error code and return console for repair |
| 2300 |  | RESERVED_0xB0 | Reserved |  |
| 2301 |  | ERROR_POST_4BL_E_VERIFY_OFFSET |  |  |
| 2302 |  | ERROR_POST_4BL_E_VERIFY_HEADER |  | Note error code and return console for repair |
| 2303 |  | ERROR_POST_4BL_E_SHA_VERIFY |  | Note error code and return console for repair |
| 2310 |  | ERROR_POST_4BL_E_LZX_EXPAND |  | Note error code and return console for repair |
| 2311 |  | ERROR_POST_4BL_E_LOAD_6BL |  | Note error code and return console for repair |
| 2312-3333 |  | RESERVED_0xB6_0xFF | Reserved |  |