Megadrive Cartridge Slot Pinout, researched written & provided by Nipedley (http://www.nipedley.net)

Key. NC – Not Connected, A – Address, D – Data, OE – Output Enable, CE – Chip Enable, Ldws/Udws – Lower/Upper Data Write Strobe, VIdent – VCC = Genesis Mode GND = Master System Mode. B31 only used for Sonic & Knuckles cartridge.

| Pin | Function | Pin | Function |
| --- | --- | --- | --- |
| A1 | GND | B1 | NC |
| A2 | VCC | B2 | NC (Hard Reset) |
| A3 | A7 | B3 | NC |
| A4 | A10 | B4 | A8 |
| A5 | A6 | B5 | A9 |
| A6 | A11 | B6 | A17 |
| A7 | A5 | B7 | A18 |
| A8 | A12 | B8 | A19 |
| A9 | A4 | B9 | A20 |
| A10 | A13 | B10 | A21 |
| A11 | A3 | B11 | A22 |
| A12 | A14 | B12 | NC |
| A13 | A2 | B13 | NC |
| A14 | A15 | B14 | NC |
| A15 | A1 | B15 | NC |
| A16 | A16 | B16 | OE |
| A17 | A0 | B17 | CS |
| A18 | GND | B18 | NC (AS) |
| A19 | D7 | B19 | NC |
| A20 | D0 | B20 | NC (DTACK) |
| A21 | D8 | B21 | NC |
| A22 | D6 | B22 | D15 |
| A23 | D1 | B23 | D14 |
| A24 | D9 | B24 | D13 |
| A25 | D5 | B25 | D12 |
| A26 | D2 | B26 | NC |
| A27 | D10 | B27 | Reset |
| A28 | D4 | B28 | Ldws |
| A29 | D3 | B29 | Udws |
| A30 | D11 | B30 | Soft Reset (VIdent) |
| A31 | VCC | B31 | Unknown Pulse (S&K) |
| A32 | GND | B32 | Cartridge Present (GND) |